Motorola Hi-Fi power amplifier

This is a very simple, low cost, Hi-Fi quality power amplifier. You can build it 5 ways, like it’s shown in the table (from 20 W to 80 W RMS).

Some comments:

- The first thing that you must do, is to measure the end transistors (T3 and T4) amplifying coefficient, the hfe or β. If their disagreement is bigger than 30 %, the amplifier would not give a clear sound. I used MJ3001 and MJ2501 transistors, and this disagreement was around 5%.

- Before the first “turning on” you must short circuit the inputs of the amp, and put a mA-meter on the output, than turn the amplifier on, and tune the R13 pot, to decrease the DC current on the output, to some uA-s, or in a lucky situation to zero. I was able to decrease it to 10 uA.

Amplifier schematic diagram:

Power supply schematic diagram:

Components and properties table:

|  | 20 / 35 W | 25 / 40 W | 35 / 60 W | 50 / 80 W | 80 / 125 W |
| --- | --- | --- | --- | --- | --- |
| T1 | BC546 |  |  |  |  |
| T2 | BC556 |  |  |  |  |
| T3 (PNP Darlington) | BD898, BD678 | BD896, BD676 | BDX34 | MJ900G | 2N6052, MJ2501 |
| T4 (NPN Darlington) | BD897, BD677 | BD895, BD675 | BDX33 | MJ1000G | 2N6059, MJ3001 |
| C1 | 10 ÷ 12 pF |  |  |  |  |
| C2 | 4,7 ÷ 5,6 nF |  |  |  |  |
| C3 (not electrolitic) | 1 ÷ 10 uF |  |  |  |  |
| C4, C5 | 2200 uF / 35 V | 4700 uF / 35 V | 4700 uF / 35 V | 4700 uF / 50 V | 4700 uF / 50 V |
| R1, R2, R9, R12 | 10 kΩ |  |  |  |  |
| R3, R4, R5, R6, R14, R15 | 3,3 kΩ |  |  |  |  |
| R7, R8 | 680 Ω |  |  |  |  |
| R10 | 100 kΩ |  |  |  |  |
| R11 | 47 kΩ |  |  |  |  |
| R13 (potenciometer) | 10 kΩ |  |  |  |  |
| F1 (fast) | 1,2 A | 1,6 A | 2 A | 2,2 A | 4 A |
| BR1 | B40C1500 | B40C2000 | B40C2800 | B40C3200 | B40C5000 |
| Cooler (Rth) | 4,5 | 2,7 | 2,0 | 1,7 | 1,5 |
| Transformer seconder voltage ( ± 20 % ) | 20 + 20 V | 20 + 20 V | 20 + 20 V | 22 + 22 V | 22 + 22 V |
| Transformer seconder current | 0,8 A | 1,2 A | 1,3 A | 1,5 A | 2,5 A |
| Power (RMS) | 20 W | 25 W | 35 W | 50 W | 80 W |
| Music power | 35 W | 40 W | 60 W | 80 W | 125 W |
| Minimal speaker impedance | 8 Ω | 8 Ω | 8 Ω | 8 Ω | 4 Ω |
| Supply voltage ( ± 20 % ) | ± 25 V | ± 25 V | ± 25 V | ± 30 V | ± 30 V |
| Maximal current | 0,54 A | 0,85 A | 1,0 A | 1,15 A | 2,0 A |
| Input voltage for Pmax output | 1,8 V | 1,5 V | 1,7 V | 1,95 V | 1,7 V |
| Input resistance | 10 kΩ |  |  |  |  |
| Frequency band (-3 dB) | 0 – 100.000 Hz |  |  |  |  |
| THD at ½ P | 0,04 % |  |  |  |  |
| THD at Psin | max 0,18 % |  |  |  |  |
| Loudness | 93 dB |  |  |  |  |

I am very contented with this amplifier. It gives a very good sound quality.

Have fun with it!

Jacint Chapo

6. 5. 2005.